MANGALORE REFINERY AND PETROCHEMICALS LIMITED (MRPL)
SOVEREIGN INDUSTRIAL MAINTENANCE REPORT: MRPL-COMP-201-A

STRICTLY CONFIDENTIAL — AIR-GAPPED ON-PREMISE SYSTEM

======================================================================

# 1. Executive Summary

This industrial maintenance report details the sovereign AI-assisted evaluation of critical equipment 'VDU Wet Gas Centrifugal Compressor' (Tag: MRPL-COMP-201-A) located in 'VDU-2 (Vacuum Distillation Unit)'. Multi-source evidence ingestion includes inspection PDFs, photographs, maintenance history spreadsheets, and local MRPL Standard Operating Procedures (SOPs).

# 2. Equipment & System Metadata

| Equipment Tag | MRPL-COMP-201-A (VDU-2 (Vacuum Distillation Unit)) |
| --- | --- |
| Evaluation Timestamp | 2026-09-10 22:44:59 |
| Selected AI Model | sovereign_rule_engine (local_fallback) |
| Hardware Mode | CPU_Only (VRAM: 0 MB) |
| Risk Level | CRITICAL |
| Confidence Score | 95.0% |

# 3. Evidence & Multi-Source Findings

Fused Evidence Text:

=== PDF INSPECTION REPORT (pump_inspection_02.pdf) ===
--- Page 1 ---
MANGALORE REFINERY AND PETROCHEMICALS LIMITED (MRPL)
ROTATING MACHINERY INSPECTION REPORT - VDU UNIT 2
=======================================================================
Equipment ID: MRPL-COMP-201-A
Equipment Description: VDU Wet Gas Multi-Stage Centrifugal Compressor
Operating Unit: VDU-2 (Vacuum Distillation Overhead Section)
Inspection Date: 2026-09-08
Inspector: J. B. Hegde (Senior Reliability Specialist, API 617 Certified)
OBSERVATIONS & VIBRATION SPECTRUM ANALYSIS:
- Thrust Bearing Active Face Temperature: 94 deg C (Max Limit: 88 deg C).
- Stage-1 Radial Shaft Vibration (Probe X): 6.8 mm/s RMS (Alarm Limit: 4.5 mm/s).
- Stage-1 Radial Shaft Vibration (Probe Y): 6.4 mm/s RMS.
- Sub-synchronous Vibration (0.43X Running Speed): Oil whirl / hydrodynamic instability detected.
- Lube Oil Differential Pressure (Delta-P): Dropped to 0.72 bar (Normal range: 1.2 - 1.6 bar).
- Dry Gas Seal Nitrogen Buffer Flow: Elevated at 18.5 Nm3/h (Normal baseline: < 10 Nm3/h).
ROOT CAUSE HYPOTHESIS:
Lube oil viscosity degradation combined with severe labyrinth seal contamination leading
to hydrodynamic journal bearing destabilization and incipient axial thrust collar scrubbing.
MAINTENANCE DIRECTIVE & SAFETY CLASS: CRITICAL (LEVEL 1)
1. Immediate load reduction on VDU-2 by 25% to throttle suction volumetric flow.
2. Emergency shutdown within 24 hours to prevent catastropic rotor-stator contact.
3. Replace tilting-pad thrust bearing shoes and perform high-pressure lube filter swap.
4. Overhaul Dry Gas Seal (DGS) cartridge #1 and restore N2 buffer differential.

=== EQUIPMENT PHOTOGRAPH & VISUAL TELEMETRY (pump_image_03.jpg) ===
Image File: pump_image_03.jpg (650x420 px, JPEG)
Visual Inspection Analysis (Heat Exchanger MRPL-HEX-105-AB):
1. Thermographic Heatmap: Severe thermal gradient across shell passes; cold stagnant zones indicate tube bundle blockage.
2. Flange & Nozzle: Weeping residue staining detected below south channel head gasket.
3. Shell Casing: Localized insulation cladding deformation due to thermal stress.

OCR Readings:
[OCR Extracted Observation from pump_image_03.jpg]:
- Equipment Tag: MRPL-HEX-105-AB (CDU Pre-Heat Shell & Tube Exchanger)
- Shell Delta-P Gauge: 2.45 bar (Clean Baseline: 0.85 bar, Limit: 1.50 bar)
- Tube Delta-P Gauge: 1.95 bar (Clean Baseline: 0.90 bar)
- Thermal IR Annotation: U-Value 215 W/m2-K (Design: 420 W/m2-K, -48% drop)
- Flange Inspection: Active crude residue weeping on Channel Head gasket

=== MAINTENANCE HISTORY SPREADSHEET (maintenance_history_02.xlsx) ===
Spreadsheet: maintenance_history_02.xlsx | Total Records: 5 | Columns: Log_ID, Date, Asset_Tag, Component, Intervention, Technician, Vibration_RMS_Post, Lube_Temp_C, Status
Recent Maintenance Records:
  Row 1: {'Log_ID': 'LOG-COMP-101', 'Date': '2025-06-12', 'Asset_Tag': 'MRPL-COMP-201-A', 'Component': 'Dry Gas Seal Cartridge', 'Intervention': 'Replaced Primary Seal Ring', 'Technician': 'V. Kulkarni', 'Vibration_RMS_Post': 2.1, 'Lube_Temp_C': 58, 'Status': 'Closed'}
  Row 2: {'Log_ID': 'LOG-COMP-142', 'Date': '2025-10-04', 'Asset_Tag': 'MRPL-COMP-201-A', 'Component': 'Lube Oil Console', 'Intervention': 'Duplex Filter Cartridge Swap', 'Technician': 'T. Nair', 'Vibration_RMS_Post': 2.4, 'Lube_Temp_C': 61, 'Status': 'Closed'}
  Row 3: {'Log_ID': 'LOG-COMP-219', 'Date': '2026-02-18', 'Asset_Tag': 'MRPL-COMP-201-A', 'Component': 'Tilting Pad Journal', 'Intervention': 'Clearance Inspection & Shimming', 'Technician': 'S. Desai', 'Vibration_RMS_Post': 3.2, 'Lube_Temp_C': 68, 'Status': 'Closed'}
  Row 4: {'Log_ID': 'LOG-COMP-308', 'Date': '2026-06-25', 'Asset_Tag': 'MRPL-COMP-201-A', 'Component': 'Coupling Diaphragm', 'Intervention': 'Laser Alignment & Bolt Torquing', 'Technician': 'V. Kulkarni', 'Vibration_RMS_Post': 3.6, 'Lube_Temp_C': 72, 'Status': 'Closed'}
  Row 5: {'Log_ID': 'LOG-COMP-390', 'Date': '2026-09-08', 'Asset_Tag': 'MRPL-COMP-201-A', 'Component': 'Thrust Bearing', 'Intervention': 'Emergency Inspection - High Temp/Vib', 'Technician': 'J. B. Hegde', 'Vibration_RMS_Post': 6.8, 'Lube_Temp_C': 94, 'Status': 'Active Alert'}

=== RELEVANT LOCAL SOP CITATIONS (RAG) ===
Source: maintenance_sop_02.pdf — Page 1
Extract: MANGALORE REFINERY AND PETROCHEMICALS LIMITED (MRPL) TECHNICAL MANUAL & SOP: CENTRIFUGAL COMPRESSOR RELIABILITY (SOP-MRPL-COMP-501) Document Version: 4.0 | Standard: API 617 8th Edition | Classification: Controlled ======================================================================= SECTION 1: VIBRATION TRIPPING CRITERIA & RUNNING CLEARANCES 1.1 Radial Shaft Vibration Thresholds (Proximity Probes): - Alert Alarm: 4.5 mm/s RMS (or 45 micrometers peak-to-peak). - Emergency Automatic Trip: 7.1 mm/s RMS (or 65 micrometers peak-to-peak). - Spectral Analysis Protocol: Any presence of sub-synchronous peaks between 0.40X and 0.48X running speed indicates oil whirl; immediately verify lube temp. SECTION 2: LUBRICATING OIL & BEARING TEMPERATURE SPECIFICATIONS 2.1 Lubricant Spec: ISO VG 46 Turbine Oil with rust/oxidation inhibitors (ASTM D4304). 2.2 Lube Oil Header Pressure: 1.8 to 2.4 bar gauge. Filter Delta-P limit: 1.0 bar max. 2.3 Bearing Temperature Limits: - Journal Bearings: Max allowable 90 deg C. Trip at 100 deg C. - Active Thrust Bearing Face: Max continuous 88 deg C. Trip at 98 deg C. SECTION 3: DRY GAS SEALS (DGS) & BARRIER SYSTEM 3.1 Buffer Gas (N2) Supply Pressure: Must be minimum 3.0 bar above internal seal cavity. 3.2 Primary vent leakage exceeding 12 Nm3/h indicates primary seal face micro-scoring. SECTION 4: TROUBLESHOOTING GUIDE (FMEA CROSS-REFERENCE) Failure Mode: High Radial Vibration + Oil Whirl. Corrective Actions: (a) Increase lube oil supply temp to 45C to reduce viscosity, (b) Verify tilting pad pivot clearances, (c) Check shaft balance & alignment.

Source: maintenance_sop_05.pdf — Page 1
Extract: MANGALORE REFINERY AND PETROCHEMICALS LIMITED (MRPL) STANDARD OPERATING PROCEDURE: SEVERE SERVICE CONTROL VALVES (SOP-MRPL-INST-204) Document Version: 2.8 | Standards: ANSI/FCI 70-2 / ISA-75.01 | Custodian: Instrumentation ======================================================================= SECTION 1: ACTUATOR TORQUE MARGINS & TIMING CRITERIA 1.1 Motor-Operated Valve (MOV) Torque Margin: - Required Available Torque Margin: >= 30% above unseating breakaway torque. - Warning Threshold: Torque margin between 20% and 29%. - Critical Threshold: Margin < 20% requires immediate actuator overhaul. 1.2 Full Stroke Time Tolerance: - Emergency Isolation Valves: Must fully close within 25.0 seconds. - Stroke time exceeding 30 seconds triggers safety loop compliance violation. SECTION 2: SEAT LEAKAGE & ACOUSTIC CAVITATION STANDARDS 2.1 Class VI Shut-Off Criteria: Zero visible bubble leakage at ambient test conditions. 2.2 Acoustic Cavitation Thresholds (Ultrasonic 30 - 45 kHz band): - Normal Non-Cavitating Operation: < 55 dB. - Mild Incipient Cavitation: 55 to 68 dB. - Severe Cavitation / Trim Erosion Risk: > 72 dB. SECTION 3: PACKING GLAND & HIGH-TEMPERATURE RESIDUE SERVICE 3.1 Gland Packing Material: High-purity flexible graphite with Inconel wire reinforcement. 3.2 Live-Loaded Belleville Springs: Check spring deflection every 6 months to maintain load.

Source: maintenance_sop_03.pdf — Page 1
Extract: MANGALORE REFINERY AND PETROCHEMICALS LIMITED (MRPL) MAINTENANCE SOP: SHELL & TUBE HEAT EXCHANGER INTEGRITY (SOP-MRPL-HEX-308) Document Version: 3.2 | Standard: TEMA Class R / API 660 | Custodian: Static Inspection ======================================================================= SECTION 1: HYDRAULIC & FOULING OPERATIONAL THRESHOLDS 1.1 Pressure Drop (Delta-P) Limits: - Clean Baseline Shell Delta-P: 0.70 - 0.90 bar. - Advisory Alert Threshold: Delta-P > 1.50 bar (Plan off-line hydro-jetting). - Critical Fouling Limit: Delta-P > 2.20 bar (Mandatory bypass diversion within 72 hrs). SECTION 2: THERMAL DEGRADATION THRESHOLDS 2.1 Overall Heat Transfer Coefficient Degradation (U/U_design): - Acceptable: > 75% of design duty. - Marginal / Clean Scheduling Required: 60% to 75%. - Severe Degradation / Unacceptable Loss: < 55% of design heat transfer. SECTION 3: HYDROSTATIC PRESSURE TESTING & LEAK DETECTION 3.1 Shell-Side Test Pressure: 1.5 x Design Pressure = 37.5 bar gauge. 3.2 Tube-Side Test Pressure: 1.5 x Design Pressure = 28.0 bar gauge. 3.3 Holding Time: Minimum 60 minutes with zero observable pressure decay. 3.4 Tube Plugging Limit: Maximum allowable plugged tubes is 10% of total count (42 tubes). SECTION 4: CHEMICAL CLEANING SAFETY PROTOCOL 4.1 Ensure full gas freeing and LEL < 0% before opening channel head cover. 4.2 Neutralize acidic residues with 2% sodium carbonate wash prior to water lance entry.

Source: sample_maintenance_sop.pdf — Page 1
Extract: MANGALORE REFINERY AND PETROCHEMICALS LIMITED (MRPL) STANDARD OPERATING PROCEDURE: CENTRIFUGAL PUMP OVERHAUL (SOP-MRPL-MECH-402) Document Version: 3.1 | Classification: Internal SOP | Custodian: Mechanical Reliability ======================================================================= SECTION 4.0: OPERATING THRESHOLDS & SAFE OPERATING WINDOWS (SOW) 4.1 Scope: Applicable to all API 610 single/multi-stage centrifugal pumps in CDU/VDU. 4.2 Bearing Temperature Limits: - Normal Continuous Range: 45 deg C to 70 deg C. - Advisory Warning Threshold: 75 deg C. - High Trip / Shutdown Mandate: Exceeding 80 deg C requires plant notification and scheduled shutdown within 48 hours to avert catastrophic shaft seizure. 4.3 Vibration Severity Criteria (ISO 10816-3 Group 1/2 Rigid Mount): - Zone A (Good): 0.0 - 2.8 mm/s RMS. - Zone B (Acceptable): 2.8 - 4.5 mm/s RMS. - Zone C (Unrestricted Operation Not Permitted / Alarm): > 4.5 mm/s RMS. - Zone D (Trip / Immediate Danger): > 7.1 mm/s RMS. 4.4 Mechanical Seal Integrity (API 682 Standard): - Normal barrier fluid leakage: < 2 drops/min. - Leakage exceeding 5 drops/min indicates primary face degradation. - Active leakage at 15 drops/min demands cartridge replacement and barrier flush. 4.5 Overhaul Intervals: Full rotating assembly overhaul every 12 months in crude service.

# 4. Knowledge Base SOP Citations

Source: maintenance_sop_02.pdf — Page 1
Relevant Extract: MANGALORE REFINERY AND PETROCHEMICALS LIMITED (MRPL) TECHNICAL MANUAL & SOP: CENTRIFUGAL COMPRESSOR RELIABILITY (SOP-MRPL-COMP-501) Document Version: 4.0 | Standard: API 617 8th Edition | Classification: Controlled ======================================================================= SECTION 1: VIBRATION TRIPPING CRITERIA & RUNNING CLEARANCES 1.1 Radial Shaft Vibration Thresholds (Proximity Probes): - Alert Alarm: 4.5 mm/s RMS (or 45 micrometers peak-to-peak). - Emergency Automatic Trip: 7.1 mm/s RMS (or 65 micrometers peak-to-peak). - Spectral Analysis Protocol: Any presence of sub-synchronous peaks between 0.40X and 0.48X running speed indicates oil whirl; immediately verify lube temp. SECTION 2: LUBRICATING OIL & BEARING TEMPERATURE SPECIFICATIONS 2.1 Lubricant Spec: ISO VG 46 Turbine Oil with rust/oxidation inhibitors (ASTM D4304). 2.2 Lube Oil Header Pressure: 1.8 to 2.4 bar gauge. Filter Delta-P limit: 1.0 bar max. 2.3 Bearing Temperature Limits: - Journal Bearings: Max allowable 90 deg C. Trip at 100 deg C. - Active Thrust Bearing Face: Max continuous 88 deg C. Trip at 98 deg C. SECTION 3: DRY GAS SEALS (DGS) & BARRIER SYSTEM 3.1 Buffer Gas (N2) Supply Pressure: Must be minimum 3.0 bar above internal seal cavity. 3.2 Primary vent leakage exceeding 12 Nm3/h indicates primary seal face micro-scoring. SECTION 4: TROUBLESHOOTING GUIDE (FMEA CROSS-REFERENCE) Failure Mode: High Radial Vibration + Oil Whirl. Corrective Actions: (a) Increase lube oil supply temp to 45C to reduce viscosity, (b) Verify tilting pad pivot clearances, (c) Check shaft balance & alignment.

Source: maintenance_sop_05.pdf — Page 1
Relevant Extract: MANGALORE REFINERY AND PETROCHEMICALS LIMITED (MRPL) STANDARD OPERATING PROCEDURE: SEVERE SERVICE CONTROL VALVES (SOP-MRPL-INST-204) Document Version: 2.8 | Standards: ANSI/FCI 70-2 / ISA-75.01 | Custodian: Instrumentation ======================================================================= SECTION 1: ACTUATOR TORQUE MARGINS & TIMING CRITERIA 1.1 Motor-Operated Valve (MOV) Torque Margin: - Required Available Torque Margin: >= 30% above unseating breakaway torque. - Warning Threshold: Torque margin between 20% and 29%. - Critical Threshold: Margin < 20% requires immediate actuator overhaul. 1.2 Full Stroke Time Tolerance: - Emergency Isolation Valves: Must fully close within 25.0 seconds. - Stroke time exceeding 30 seconds triggers safety loop compliance violation. SECTION 2: SEAT LEAKAGE & ACOUSTIC CAVITATION STANDARDS 2.1 Class VI Shut-Off Criteria: Zero visible bubble leakage at ambient test conditions. 2.2 Acoustic Cavitation Thresholds (Ultrasonic 30 - 45 kHz band): - Normal Non-Cavitating Operation: < 55 dB. - Mild Incipient Cavitation: 55 to 68 dB. - Severe Cavitation / Trim Erosion Risk: > 72 dB. SECTION 3: PACKING GLAND & HIGH-TEMPERATURE RESIDUE SERVICE 3.1 Gland Packing Material: High-purity flexible graphite with Inconel wire reinforcement. 3.2 Live-Loaded Belleville Springs: Check spring deflection every 6 months to maintain load.

Source: maintenance_sop_03.pdf — Page 1
Relevant Extract: MANGALORE REFINERY AND PETROCHEMICALS LIMITED (MRPL) MAINTENANCE SOP: SHELL & TUBE HEAT EXCHANGER INTEGRITY (SOP-MRPL-HEX-308) Document Version: 3.2 | Standard: TEMA Class R / API 660 | Custodian: Static Inspection ======================================================================= SECTION 1: HYDRAULIC & FOULING OPERATIONAL THRESHOLDS 1.1 Pressure Drop (Delta-P) Limits: - Clean Baseline Shell Delta-P: 0.70 - 0.90 bar. - Advisory Alert Threshold: Delta-P > 1.50 bar (Plan off-line hydro-jetting). - Critical Fouling Limit: Delta-P > 2.20 bar (Mandatory bypass diversion within 72 hrs). SECTION 2: THERMAL DEGRADATION THRESHOLDS 2.1 Overall Heat Transfer Coefficient Degradation (U/U_design): - Acceptable: > 75% of design duty. - Marginal / Clean Scheduling Required: 60% to 75%. - Severe Degradation / Unacceptable Loss: < 55% of design heat transfer. SECTION 3: HYDROSTATIC PRESSURE TESTING & LEAK DETECTION 3.1 Shell-Side Test Pressure: 1.5 x Design Pressure = 37.5 bar gauge. 3.2 Tube-Side Test Pressure: 1.5 x Design Pressure = 28.0 bar gauge. 3.3 Holding Time: Minimum 60 minutes with zero observable pressure decay. 3.4 Tube Plugging Limit: Maximum allowable plugged tubes is 10% of total count (42 tubes). SECTION 4: CHEMICAL CLEANING SAFETY PROTOCOL 4.1 Ensure full gas freeing and LEL < 0% before opening channel head cover. 4.2 Neutralize acidic residues with 2% sodium carbonate wash prior to water lance entry.

Source: sample_maintenance_sop.pdf — Page 1
Relevant Extract: MANGALORE REFINERY AND PETROCHEMICALS LIMITED (MRPL) STANDARD OPERATING PROCEDURE: CENTRIFUGAL PUMP OVERHAUL (SOP-MRPL-MECH-402) Document Version: 3.1 | Classification: Internal SOP | Custodian: Mechanical Reliability ======================================================================= SECTION 4.0: OPERATING THRESHOLDS & SAFE OPERATING WINDOWS (SOW) 4.1 Scope: Applicable to all API 610 single/multi-stage centrifugal pumps in CDU/VDU. 4.2 Bearing Temperature Limits: - Normal Continuous Range: 45 deg C to 70 deg C. - Advisory Warning Threshold: 75 deg C. - High Trip / Shutdown Mandate: Exceeding 80 deg C requires plant notification and scheduled shutdown within 48 hours to avert catastrophic shaft seizure. 4.3 Vibration Severity Criteria (ISO 10816-3 Group 1/2 Rigid Mount): - Zone A (Good): 0.0 - 2.8 mm/s RMS. - Zone B (Acceptable): 2.8 - 4.5 mm/s RMS. - Zone C (Unrestricted Operation Not Permitted / Alarm): > 4.5 mm/s RMS. - Zone D (Trip / Immediate Danger): > 7.1 mm/s RMS. 4.4 Mechanical Seal Integrity (API 682 Standard): - Normal barrier fluid leakage: < 2 drops/min. - Leakage exceeding 5 drops/min indicates primary face degradation. - Active leakage at 15 drops/min demands cartridge replacement and barrier flush. 4.5 Overhaul Intervals: Full rotating assembly overhaul every 12 months in crude service.

# 5. Condition Reasoning & Action Recommendation

INDUSTRIAL CONDITION REASONING (COMPRESSOR MRPL-COMP-201-A):
1. Multimodal Evidence: Stage-1 radial vibration reaches 6.8 mm/s RMS with active sub-synchronous whirl (0.43X), while thrust bearing active face operates at 94 deg C (Max limit: 88 deg C).
2. Telemetry & Lube Console: Lube oil differential pressure dropped to 0.72 bar (Normal: 1.2 - 1.6 bar), indicating severe filter blinding or foaming in the lube skid.
3. SOP Alignment: Per SOP-MRPL-COMP-501 / API 617, radial vibration exceeding 4.5 mm/s RMS with sub-synchronous whirl indicates hydrodynamic bearing instability; operation > 7.1 mm/s mandates emergency trip.
4. Conclusion: Imminent risk of axial thrust collar wipeout and rotor-stator contact under continuous VDU-2 operation.

CRITICAL ACTION DIRECTIVE (MRPL-COMP-201-A):
1. Throttle VDU-2 suction rate by 25% immediately to relieve compressor aerodynamic loading.
2. Schedule controlled unit shutdown within 24 hours to prevent thrust collar seizure.
3. Perform complete duplex lube oil filter changeout and lube temperature re-trimming to 45 deg C.
4. Inspect tilting pad thrust shoes and replace Dry Gas Seal Cartridge #1.

# 6. Human Approval & Decision Audit Stamp

Approval Status : APPROVED

Approver Input  : Verified by MRPL Operations Manager.

Audit Status    : VERIFIED & SEALED LOCAL DELIVERABLE